7. Predictive coding
Objective: to compress an image by exploiting the high-level of redundancy that exists in most images. This can be done by trying to predict the current pixel value in an image using the previous ones.

When pixel values are predicted, errors are made. Therefore only the prediction errors have to be transmitted.

· To transmit the prediction error, three basic data compression techniques are often used:

Run-length encoding: runs of data (sequences in which many consecutive elements have the same data value) are represented by a single symbol.

Variable length coding: it is a statistical coding technique. It uses short codeword to represent values which occur frequently and long codeword to represent the values which occur less frequently.
Nonuniform quantification:

[image: image1.png]out

d t

A
L/2 symbols




· For a binary image (or an image with few levels), a prediction pattern can be used. A table gives the prediction to make for all possible stated of this pattern.

· For a gray-level image, the predictor is a function of a pixel’s neighbors.

Some possible predictors:
B C

A x

· 
[image: image2.wmf]3

/

3

/

3

/

ˆ

C

B

A

x

+

+

=


· 
[image: image3.wmf]C

B

A

x

25

.

0

25

.

0

5

.

0

ˆ

+

+

=


· 
[image: image4.wmf]î

í

ì

-

>

-

=

otherwise

C

C

B

B

A

A

x

ˆ


[image: image5.png]slx.y1




[image: image6.png]



· Only the prediction error (which has a smaller dynamic range) is transmitted. For higher compression, this error should be quantized.


[image: image7.wmf][

]

)

(

ˆ

)

(

)

(

n

u

n

u

Q

n

e

q

q

-

=




Important: When the error is quantized, the encoder and the decoder must use the same values for prediction!

[image: image8.png]ug ()

Quantizer

Predictor

Predictor





Motion-compensated coding
Objective: to predict an image in a sequence using the previous image. Since there is motion in the sequence, the prediction uses motion vectors.

Motion estimation is based on block matching: we must find in the previous image, the most similar block, i.e. the one that minimizes:


[image: image9.wmf]å

å

+

+

-

=

-

y

x

t

t

v

y

u

x

I

y

x

I

D

2

1

))

,

(

)

,

(

(


[image: image10.png]best block matching

search window

B
s

- displacement vector

cortelation window

An original block




from: http://www-ec.njit.edu/~shi/courses/ECE789/motion.pdf

[image: image11.png]( Motion-compensated
displacement vectors Prediction error





from: http://www.stanford.edu/class/ee368b/Handouts/17-MotionCompensation.pdf

MPEG: Moving Picture Experts Group

Objective: to define a coding standard that can be applied on image sequences. 

Main characteristics:

· Non-sequential access

· Fast forward and rewind

· Fast decoding (<150ms)

· Encoding does not have to be done in real-time

· Flexible

Techniques: 

· Motion-compensated predictive coding: on 16x16 macro-blocks

· DCT: 8x8 blocks

· 1 macro-block= 4 luminance block + 2 chrominance blocks

[image: image12.png]Juminance

|

Tum|lum

.
chrominance




from: http://rnvs.informatik.tu-chemnitz.de/~jan/MPEG/HTML/mpeg_tech.html
3 types of frames:

· Intraframe (I): access point, low compression (DCT)

· Predicted frame (P): using a past frame.

· Interpolated frame (B): using past and future frames.

[image: image13.png]MPEG display order

-
1 BBBPUBBB P

—= forward prediction of P —frames

— forward prediction of B—frames
— backward prediction of B—frames




from: http://rnvs.informatik.tu-chemnitz.de/~jan/MPEG/HTML/mpeg_tech.html
The frames of the illustrated sequence will be transmitted in the following order: IPBBBPBBB

Approximatly 12 P and B frames between each I frames.

The standard does not define how to obtain the motion vectors.

Compression rate: between 1/16 and 1/40

CCIR601: 720x486x30Hz 
=> 
5 to 10 Mbps






I= 150Kbits, P= 50Kbits, B= 20Kbits

[image: image14.png]I-frame

%

motion vector

-

forward prediction

-

prediction error compensation

resulting picture





from: http://rnvs.informatik.tu-chemnitz.de/~jan/MPEG/HTML/mpeg_tech.html
[image: image15.jpg]« Motion Estimation

&t

[ &

Gea4E o s s 62| [BHe2 el 5868381 | (42 05 00 011
ssa2mmmm msl | (2ee 352838381 | |4 0-10 000 0
328284 5787378184 | |e3m2m4E7E7a78188 | |1 0 0 00 004
51565 s s2 528457 |- Bl BB B E2828487 (-0 -1 -1-1 0 0 0 0
3184 8785w sl | |asiesEress9aesl( [0 0200040
318585581 w9 a1ss| |mass ssla9slas| |1 04-2000 2
52515 53 95 59 8154 | |5l &7 8658386598184 [1 6 000000

o 1 mmassam| D62 no1an





from: http://www.cmlab.csie.ntu.edu.tw/cml/dsp/training/coding/mpeg1/

Intra frame

[image: image16.jpg]B
Ineg | —] .
[op—

o | reashrion
v
o« FHH
vic D DRCH kit
° Framl<im P
—
- ACRLE





Inter frame

[image: image17.png]Imsge

REB wYChEr
Anasowmpe |y

v

och

>

Pariiicn o Macroblucks

zm..._.m..k

Oc:

ane macroblock

m.mmi,

Cu] [Cr
¥ChCrmal:1

Clpredict (] motion

error mm
o MR ,,,m
Yrerac o

Onc">0%

sev Ly [] a0 o, o




Motion compensated prediction coding:

[image: image18.png]buffer fullness
Control
prediction
rror P
input_frame
e uantise Entropy

—
postile] [l
i
] e e [
frame
-
[ U aton 1. e
frame
v
oo
] Elare [ tecoded




from: http://cnx.rice.edu/content/m11142/latest/

_1162626245.unknown

_1162627447.unknown

_1162633231.unknown

_1162626481.unknown

_1162626234.unknown

