Jean-Philippe Bergeron	4656474

# Questions

# Shared Memory Systems

## Questions

# 1.

Draw a bus based shared memory system with 3 processor and 2 memories.

# 2.

What are the 2 right requirements for shared memory system

  - Have multiple memory modules

  - Support for memory coherency

  - Support for atomic operations on data

  - Communicate from processor to processor

# 3.

What is an atomic operation?

An operation that can't be divided or interrupted.

# 4.

By using Load-Locked(LL) and Store-Conditional(SC) in the following code, in which condition can sc fail. Give an example.

lock:  	ll,reg1, location	/* LL location to reg1 */

bnz	reg1, lock	/* if location is locked, try again*/

sc	location, reg2	/* SC reg2 into location*/

beqz	lock	/* if failed, start again */

ret		/* return control to the caller of lock */

It can fail in case another processor gets the lock after LL returns that the lock is unlocked.

# 5.

In a system with two processor and 2 cache. If there is no cache coherence protocol, can one processor read an old value in the case of

- Write-through cache yes

- Write-back cache yes

## Problems

# 1.

Convert this loop into a parallel program using INITIALIZE(), LOCK(), UNLOCK() or BARRIER(). All processor starts at the same time.

| INITIALIZE | assigns a number (proc_num) to each processor in the system; assigns the total number of processors (num_procs). |
| --- | --- |
| LOCK(data) | Allows a processor to “check out” a certain piece of shared data. While one processor has the data locked, no other processors can obtain the lock. The lock is blocking, so once a LOCK is encountered, execution of the program cannot proceed until the LOCK is obtained. |
| UNLOCK(data) | releases a lock so that other processors can obtain it. |
| BARRIER(n_procs) | When a BARRIER is encountered, a processor waits at that BARRIER until n_procs processors reach the BARRIER, then execution can proceed. |

int global_sum = 0;

int local_sum = 0;

for (int i=0; i< 1024; i++){

for (int j=0; j< 1024; j++){

local_sum += j;

}

}

global_sum += local_sum;

printf("sum is %d", global_sum);

Is the barrier essential?

Answer

INITIALIZE;

int global_sum = 0; // can be done by all thread

int local_sum = 0;

lower_ind = (1024 / num_procs) * proc_num;

upper_ind = (1024 / num_procs) * (proc_num + 1);

for (int i= lower_ind; i< upper_ind; i++){

for (int j=0; j< 1024; j++){

local_sum += j;

}

}

LOCK(global_sum);

global_sum += local_sum;

UNLOCK(global_sum);

BARRIER(num_procs);

printf("sum is %d", global_sum);

Yes, global_sum has to be computed before the printf can be executed.

# 2.

Assume the following atomic function for swap. Design a locking function using swap.

int swap ( int* register, int oldval, int newval)

{

int old_reg_val = *register;

if (old_reg_val == oldval)

*register = newval;

return old_reg_val;

}

make a lock and unlock functions using swap

void lock(int* register) {

while (swap(register,0,1) ==1 ) {}

}

void unlock(int* register) {

*register = 0;

}

# 3.

Describe the functionality of test&set and how can it be used to implement a lock?

The function writes one to the memory location, it returns the value that was in the memory before the set.

2 possible solution

lock:	t&s	register, location

bnz	lock	 		/* if not 0, try again */

ret				/* return control to caller */

or

while (t&s(location) !=0) {}

# 4.

How would you implement the test&set function in C? Assume your function will be executed atomically.

int test&set(*location){

int previous_value = *location;

*location = 1;

return previous_value;

}

# 5.

Where should barriers be used in the following program and why?

INITIALIZE; //assign proc_nums and num_procs

if (proc_num == 0) { //initialize the sum

read_array(global_array_to_sum, size); //read the array and array size from file

LOCK(global_sum);

global_sum = 0;

UNLOCK(global_sum);

}

local_sum = 0;

size_to_sum = size/num_procs;

lower_ind = size_to_sum * proc_num;

upper_ind = size_to_sum * (proc_num + 1);

BARRIER(num_procs); //answer 1

for (int i = lower_ind; i < upper_ind; i++)

local_sum += global_array_to_sum[i];

LOCK(global_sum); //locks the sum variable so only this process can change it

global_sum += local_sum;

UNLOCK(global_sum); //gives the sum back so other procs can add to it

BARRIER(num_procs); //answer 2

if (proc_num == 0)

printf("sum is %d", global_sum);

The barrier at place answer1 is needed because global_array_to_sum has to be read by processor 0 before it can be used.

The barrier at place answer2 is needed because the variable global_sum has to be computed before it can be displayed.