Université d’Ottawa      ·     University of Ottawa

École d'ingénierie et de technologie			School of Information Technology

de l'information (EITI)				and Engineering (SITE)

CEG 4131 Computer Architecture III: QUIZ 2

| Date: October 14th | Professor: Dr. M. Bolic |
| --- | --- |
| Duration: 30minutes | Session: Fall 2005-2006 |
| Total Points = 100 |  |

Note: Closed book exam. Cheat-sheets are not allowed. Calculators are allowed.

Name: _______________________Student ID:_______________

QUESTION #1 (a, b, c, d 10 points, total 40)

Only one answer is correct a, b, or c. Choose one answer?

- The following network is asymmetric:

  - Hypercube

  - 2D Mesh

  - Ring

- If the main concern in the design of the interconnection network is configurability (ability to easily add more nodes), then which multistage network should be used:

  - Bus

  - Omega network

  - Crossbar network

I will accept both A and B here. It is easiest to add a node to the bus. If we want to add the node to the crossbar with nxn switches then we have to add additional 2n-1 switches. B is correct since the Omega network is a multistage network that is easier to add a node.

- The number of permutations in 8x8 crossbar network is

  - 256

  - 40320

  - 16777216

- In a single-bus system that uses split transactions

  - Both masters and slaves have to compete for the bus by arbitration

  - Only the masters have to compete for the bus by arbitration

  - No arbitration is necessary

QUESTION #2 (a, b, c 20 points, total 60 points)

- Compute the diameter and bisection width for 3D mesh with p=64 processors.

P=64 processors means that there are 4 processors in one dimension. Bisection width is 4*4=16 and D=9.

- Compute the diameter and bisection width for 2D torus with p=64 processors (assume that routing is bidirectional).

The number of nodes in one dimension is 8.

D of the ring is 4, so that D of the torus is 8. Bisection width is 2*8=16

c. Construct an 8-input Omega network using 2 x 2 switch modules in multiple stages. Show the routing of the message from input 010 to output 110.